CYBERSECURITY WORKSHOP — INSTRUCTOR NOTES

Day 5: Networking Fundamentals II

DNS · DHCP · NAT · Subnetting  —  Hands-on: DNS Enumeration & Packet Analysis (Kali Linux)

## Session Overview

| Duration | Format | Environment |
| --- | --- | --- |
| 3 hours (1.5 hr theory + 1.5 hr hands-on) | Lecture + guided lab | Kali Linux VM (attacker), target host/router on same LAN |

## Learning Objectives

By the end of Day 5, students should be able to:

- Explain how DNS resolves domain names to IP addresses, and identify common DNS record types.

- Describe the DHCP DORA process and how hosts obtain IP configuration automatically.

- Explain NAT (Static, Dynamic, PAT/Overload) and why it matters for private-to-public IP translation.

- Calculate subnets and subnet masks using CIDR notation for a given network requirement.

- Perform basic DNS enumeration using command-line and automated tools on Kali Linux.

- Capture and analyze DNS and DHCP traffic using Wireshark and tcpdump.

# PART A — THEORY (1.5 hrs)

## 1. DNS — Domain Name System

DNS is the system that translates human-friendly domain names (like google.com) into machine-friendly IP addresses (like 142.250.193.14). Think of it as the phonebook of the internet.

How DNS Resolution Works (step by step)

- User types google.com into a browser.

- The OS checks its local DNS cache — if found, resolution ends here.

- If not cached, the request goes to a Recursive Resolver (usually your ISP or a public resolver like 8.8.8.8).

- The resolver queries a Root Server → which points to the correct TLD Server (.com).

- The TLD server points to the Authoritative Name Server for google.com.

- The authoritative server returns the actual IP address.

- The resolver caches the result and returns it to the user's browser.

Teaching Tip Draw this as a flow diagram on the whiteboard: Client → Resolver → Root → TLD → Authoritative → back to Client. Students remember DNS far better as a diagram than as a list.

Common DNS Record Types

| Record | Purpose | Example |
| --- | --- | --- |
| A | Maps a hostname to an IPv4 address | example.com → 93.184.216.34 |
| AAAA | Maps a hostname to an IPv6 address | example.com → 2606:2800:220:1:: |
| CNAME | Alias — points one name to another name | www.example.com → example.com |
| MX | Mail server responsible for the domain | example.com → mail.example.com (priority 10) |
| NS | Authoritative name servers for the domain | example.com → ns1.example.com |
| TXT | Free-text data — SPF, DKIM, domain verification | v=spf1 include:_spf.google.com ~all |
| SOA | Start of Authority — admin & zone info | Primary NS, admin email, serial, refresh |
| PTR | Reverse lookup — IP to hostname | 34.216.184.93.in-addr.arpa → example.com |

Recursive vs Iterative Queries

- Recursive query: the resolver does all the work and returns a final answer to the client.

- Iterative query: each server returns the best answer it has (often a referral) and the resolver keeps asking the next server.

Why Attackers Care About DNS

- DNS enumeration reveals subdomains, mail servers, and infrastructure — useful in reconnaissance.

- Misconfigured DNS zones can leak internal hostnames (zone transfer / AXFR).

- DNS is also abused for attacks like DNS spoofing, cache poisoning, and DNS tunneling for data exfiltration.

## 2. DHCP — Dynamic Host Configuration Protocol

DHCP automatically assigns IP addresses and network configuration (subnet mask, gateway, DNS servers) to devices joining a network, so administrators don't have to configure every host manually.

The DORA Process

| Step | Message | Direction | Meaning |
| --- | --- | --- | --- |
| 1 | DISCOVER | Client → Broadcast | Client broadcasts: "Is there a DHCP server on this network?" |
| 2 | OFFER | Server → Client | Server replies with a proposed IP address and lease terms |
| 3 | REQUEST | Client → Broadcast | Client broadcasts acceptance of the offered IP (so other DHCP servers know it was declined) |
| 4 | ACK | Server → Client | Server confirms the lease; client configures its interface |

Teaching Tip Say it out loud with students as a mnemonic: "Do You Really Ask" = Discover, Offer, Request, Acknowledge (DORA).

Key DHCP Concepts

- Lease time: how long a client is allowed to use an assigned IP before it must renew.

- DHCP scope: the range of IP addresses a DHCP server is allowed to hand out.

- DHCP relay agent: forwards DHCP broadcasts across subnets/VLANs to a central DHCP server.

- Uses UDP — server listens on port 67, client listens on port 68.

- Security note: rogue DHCP servers on a network can hand out malicious gateway/DNS settings — this is a classic MITM technique students should be aware of.

## 3. NAT — Network Address Translation

NAT allows multiple devices on a private network (using private IP ranges) to share one or a few public IP addresses to access the internet. It conserves public IPv4 addresses and hides internal network structure.

Private IP Ranges (RFC 1918)

| Range | CIDR | Typical Use |
| --- | --- | --- |
| 10.0.0.0 – 10.255.255.255 | 10.0.0.0/8 | Large enterprise networks |
| 172.16.0.0 – 172.31.255.255 | 172.16.0.0/12 | Medium networks |
| 192.168.0.0 – 192.168.255.255 | 192.168.0.0/16 | Home / small office networks |

Types of NAT

- Static NAT: one private IP is permanently mapped to one public IP. Used when a server needs to be consistently reachable from outside (e.g. a web server).

- Dynamic NAT: a pool of public IPs is shared among many private IPs, assigned on a first-come basis.

- PAT (Port Address Translation) / NAT Overload: the most common form — many private IPs share a single public IP, distinguished by port numbers. This is what your home router does.

Example (PAT): 192.168.1.10:51423 --\ 192.168.1.11:51500 ---> 203.0.113.5:XXXXX ---> Internet 192.168.1.12:51610 --/ The router tracks (private IP:port) <-> (public IP:port) in a NAT table so return traffic is sent back to the correct internal host.

Why NAT Matters for Security

- NAT is not a firewall, but it provides some obscurity — internal hosts aren't directly addressable from the internet.

- Pentesters/red teamers need to understand NAT when pivoting from an external-facing host into an internal private network.

## 4. Subnetting Concepts

Subnetting divides a large network into smaller, manageable sub-networks. It improves security (isolating segments), reduces broadcast traffic, and makes efficient use of IP address space.

CIDR Notation Refresher

An IP address like 192.168.1.0/24 means the first 24 bits are the network portion, and the remaining 8 bits are for hosts.

| CIDR | Subnet Mask | Total Addresses | Usable Hosts |
| --- | --- | --- | --- |
| /24 | 255.255.255.0 | 256 | 254 |
| /25 | 255.255.255.128 | 128 | 126 |
| /26 | 255.255.255.192 | 64 | 62 |
| /27 | 255.255.255.224 | 32 | 30 |
| /28 | 255.255.255.240 | 16 | 14 |
| /30 | 255.255.255.252 | 4 | 2 (point-to-point links) |

| Subnet | Network Address | Usable Host Range | Broadcast Address |
| --- | --- | --- | --- |
| 1 | 192.168.1.0/26 | 192.168.1.1 – 192.168.1.62 | 192.168.1.63 |
| 2 | 192.168.1.64/26 | 192.168.1.65 – 192.168.1.126 | 192.168.1.127 |
| 3 | 192.168.1.128/26 | 192.168.1.129 – 192.168.1.190 | 192.168.1.191 |
| 4 | 192.168.1.192/26 | 192.168.1.193 – 192.168.1.254 | 192.168.1.255 |

Classroom Exercise Ask students to subnet 10.10.0.0/22 into 8 equal subnets on paper before showing the answer. This reinforces the bit-borrowing logic rather than rote memorization.

# PART B — HANDS-ON LAB (1.5 hrs) — Kali Linux

Environment: Kali Linux VM with internet/LAN access. Run commands as a normal user unless root is required. Ensure students only enumerate domains/hosts they own or are explicitly authorized to test (e.g., a lab-provided target or their own domain).

Ethics & Authorization Reminder Emphasize before starting: DNS enumeration against third-party domains without permission may violate terms of service or law. Use lab-designated targets, a personal domain, or authorized scope only.

## Lab 1 — DNS Enumeration

Step 1: Basic Lookups with dig, nslookup, host

$ dig example.com A $ dig example.com MX $ dig example.com NS $ dig example.com ANY $ nslookup example.com $ host example.com $ host -t txt example.com

Ask students to identify: the A record IP, the authoritative name servers, and any TXT records (often reveal SPF/verification info).

Step 2: Reverse DNS Lookup

$ dig -x 8.8.8.8 $ host 8.8.8.8

Step 3: Zone Transfer Attempt (AXFR)

A misconfigured DNS server may allow anyone to request a full copy of its zone file — a serious information leak.

$ dig axfr @ns1.example.com example.com

Most modern servers will refuse this ("Transfer failed"), which is the expected — and correct — secure behavior. Discuss why this matters.

Step 4: whois Lookup

$ whois example.com

Shows registrar, registration dates, and (if not privacy-protected) registrant contact info.

Step 5: Automated Enumeration Tools

| Tool | Purpose | Example Command |
| --- | --- | --- |
| dnsenum | Automated DNS info gathering, subdomain brute-force, zone transfer attempts | dnsenum example.com |
| dnsrecon | DNS recon with multiple scan modes (std, brute, axfr) | dnsrecon -d example.com -t std |
| fierce | Subdomain discovery via DNS brute force | fierce --domain example.com |
| sublist3r | Subdomain enumeration via search engines + DNS | sublist3r -d example.com |

# If tools are not pre-installed on the lab image: $ sudo apt update && sudo apt install dnsenum dnsrecon fierce -y

Lab 1 Deliverable

- Each student records: target domain, A record, NS records, MX records, whether zone transfer succeeded, and any subdomains discovered.

- Short discussion: which findings would be useful to an attacker during reconnaissance, and why?

## Lab 2 — Packet Analysis (DNS & DHCP)

Step 1: Capture DNS Traffic with Wireshark

- Open Wireshark on Kali, select the active interface (eth0 / wlan0).

- Apply the display filter: dns

- In a terminal, generate traffic: dig example.com

- Observe the DNS Query and DNS Response packets in Wireshark.

- Expand the DNS layer — identify Transaction ID, Query name, Query type, and Answer section.

Wireshark filter examples: dns # all DNS traffic dns.qry.name == "example.com" # queries for a specific domain dns.flags.response == 1 # only DNS responses

Step 2: Capture DNS with tcpdump (CLI alternative)

$ sudo tcpdump -i eth0 udp port 53 -n -v

Ask students to run this in one terminal, then trigger a lookup (dig/nslookup) in another, and match the request/response pair by observing source/destination IP and port 53.

Step 3: Capture DHCP Traffic

DHCP is broadcast-based, so it's best observed when a client actually requests/renews an address.

Wireshark filter: bootp (older Wireshark versions) dhcp (newer Wireshark versions — same protocol, DHCP is built on BOOTP) tcpdump equivalent: $ sudo tcpdump -i eth0 udp port 67 or udp port 68 -n -v

- Release and renew the DHCP lease to generate traffic: sudo dhclient -r eth0 (release) then sudo dhclient eth0 (renew).

- Observe the four DORA packets in Wireshark: DHCP Discover, Offer, Request, ACK.

- Click each packet and identify: Client MAC address, Offered IP, Lease Time, Server Identifier (in the DHCP options).

Teaching Tip This is a great moment to tie theory to practice — pause and have students literally point at the Discover/Offer/Request/ACK packets on screen and match them to the DORA steps from Part A.

Lab 2 Deliverable

- Screenshot or note: one DNS query/response pair with Transaction ID matched.

- Screenshot or note: all four DHCP DORA packets with the offered IP and lease time identified.

- One paragraph: what would look abnormal in this capture if a rogue DHCP server were present on the network?

# Recap & Assignment

Quick Recap Questions (ask the class)

- What are the 4 steps of DHCP, in order?

- What's the difference between an A record and a CNAME record?

- Why does PAT let one public IP serve many internal devices?

- How many usable hosts are there in a /27 subnet?

- Why is a successful AXFR zone transfer considered a security misconfiguration?

End of Day 5 Notes